20CYS384 – APES Lab

  Implementation Report: Customizing the Linux Network Stack

K.Kartheek

CH.SC.U4CYS23017

M.Avinash

CH.SC.U4CYS23024

P.Deepak Sai Vighnesh

CH.SC.U4CYS23032

Project Title

Implementation of a TCP Connection Logger through Linux Kernel Modification

1. Introduction

The Linux operating system provides a highly modular and customizable kernel that allows developers to modify system-level behavior. This project focuses on modifying the Linux kernel network stack to log TCP connections at the kernel level.

The objective is to intercept the TCP connection establishment process and log relevant details such as the source IP address and destination port. This is achieved by modifying the kernel source code and recompiling the kernel.

This project demonstrates how operating systems internally handle network communication and how developers can extend or customize kernel behavior.

2. Understanding TCP and Kernel Networking

TCP (Transmission Control Protocol) is a reliable, connection-oriented protocol used for communication over networks. Before data transmission begins, TCP establishes a connection using a three-way handshake:

* The client sends a SYN packet

* The server responds with SYN-ACK

* The client replies with ACK

Once this handshake is completed, a connection is established.

In the Linux kernel, this process is handled by various functions in the networking subsystem. One important function is:

tcp_v4_syn_recv_sock()

This function is responsible for creating a new socket when a TCP connection request is accepted. By inserting custom code in this function, we can log every successful TCP connection.

3. Downloading the Linux Kernel Source

Before modifying the kernel, the source code must be downloaded.

The following command is used:

wget https://cdn.kernel.org/pub/linux/kernel/v6.x/linux-6.19.9.tar.xz

This command downloads the Linux kernel source archive from the official kernel repository.

Next, the archive is extracted:

tar -xvf linux-6.19.9.tar.xz

This creates a directory named `linux-6.19.9`, which contains the complete kernel source code.

Time taken: Approximately 2–5 minutes depending on system performance.

4. Installing Required Dependencies

Before building the kernel, necessary development tools and libraries must be installed:

sudo apt update

sudo apt install build-essential bison flex libncurses-dev libssl-dev libelf-dev bc -y

Explanation:

* `build-essential` → provides compiler tools like GCC and Make

* `bison` and `flex` → required for parsing kernel code

* `libncurses-dev` → required for configuration menus

* `libssl-dev` → cryptographic support

* `libelf-dev` → required for kernel modules

* `bc` → used for mathematical calculations during build

Time taken: 5–10 minutes

5. Preparing the Kernel Source

Navigate into the kernel source directory:

cd linux-6.19.9

To ensure a clean build environment, the following command is used:

make mrproper

This removes any previous build files and resets the source tree.

Time taken: 1–2 minutes

6. Configuring the Kernel

Instead of using default configuration, the existing system configuration is copied:

cp /boot/config-$(uname -r) .config

This ensures that all necessary drivers (especially disk and filesystem drivers) are included.

Then update the configuration:

make oldconfig

This updates the configuration file based on the current kernel version while preserving compatibility.

Time taken: 2–3 minutes

7. Modifying the Kernel Source Code

The file responsible for TCP IPv4 operations is:

net/ipv4/tcp_ipv4.c

This file contains the implementation of TCP connection handling.

Open the file using an editor:

vim net/ipv4/tcp_ipv4.c

Inside the function `tcp_v4_syn_recv_sock()`, the following code is added:

if (newsk && req) {

printk(KERN_INFO "CustomKernel: TCP connection established from %pI4 to port %u\n",

&inet_rsk(req)->ir_rmt_addr,

ntohs(inet_sk(newsk)->inet_dport));

}

Explanation:

* This code logs the source IP address and destination port whenever a TCP connection is established.

* `printk()` is used for kernel logging.

* `%pI4` formats the IP address correctly.

* `ntohs()` converts network byte order to host byte order.

Time taken: 5 minutes

8. Compiling the Kernel

The kernel is compiled using:

make -j1

Explanation:

* `make` compiles the kernel source code.

* `-j1` ensures only one process runs at a time, preventing memory issues in a virtual machine.

Time taken: 30–60 minutes

9. Installing the Kernel

After compilation, the kernel and modules are installed:

sudo make modules_install

sudo make install

Explanation:

* `modules_install` installs kernel modules into `/lib/modules`

* `make install` installs the kernel into `/boot`

Time taken: 5–10 minutes

10. Updating Bootloader

sudo update-grub

This updates the GRUB bootloader to include the new kernel.

Time taken: ~1 minute

11. Rebooting the System

sudo reboot

After reboot, the new kernel is selected from the GRUB menu.

12. Verification of the Implementation

Check kernel version:

uname -a

Start a test server:

python3 -m http.server 8080

Trigger a TCP connection:

curl localhost:8080

Check kernel logs:

dmesg | grep CustomKernel

Expected output:

CustomKernel: TCP connection established from 127.0.0.1 to port 8080

13. Challenges Faced

* Missing dependencies during compilation

* Read-only filesystem issues

* Kernel boot failure (initramfs error)

* Memory limitations in virtual machine

* Incorrect configuration using default settings

14. Conclusion

This project successfully demonstrated how to modify the Linux kernel to alter network behavior. By inserting logging functionality into the TCP connection process, we gained a deeper understanding of kernel internals and networking.

The project highlights the importance of correct configuration, dependency management, and system-level debugging.

15. References

* Linux Kernel Documentation

* Kernel Source Code

* TCP/IP Networking Concepts